2. SYSTEM ANALYSIS

  - CURRENT SYSTEM:

Previously, Max-Delta has been proposed to efficiently infer the underlay topology among a group of hosts. It divides the inference process into multiple iterations. In each iteration, a central server collects the trace route results from the hosts. The server then selects some representative paths for the hosts to trace route in the next iteration and aims at revealing as much undiscovered information on the underlay as possible.

The simulation results on Transit-Stub topologies have shown that in a group of 256 hosts, each host only needs to trace route around 10 to 14 other hosts to discover 95 percent of underlay links. This reduces the number of trace routes by 62 percent as compared to a random measurement method and by 89 percent as compared to the full measurement method.

A limitation of Max-Delta is that the central server may not be able to support a large group.

  - PROPOSED SYSTEM

In proposed distributed inference scheme based on Max-Delta scheme, each host joins an overlay tree before topology inference. A host then conducts trace routes and distributes the results to all the other hosts through the overlay tree. As a result, each host can receive trace route results from other hosts and maintain a partially discovered topology. Based on that, a host can use Max-Delta to select trace route targets by it and continue conducting trace routes.

We have studied an important issue in the scheme, that is, how the overlay tree can be constructed so that hosts can quickly exchange trace route results. Since every host sends out trace route results to others, we need to build a source-unspecific low-diameter overlay tree under certain degree bounds.

Moreover the following algorithm has been proposed; a distributed tree construction algorithm based on the out degree bounds of hosts. It first identifies a host as the tree root and then inserts new incoming hosts around the root. A host with a larger out degree bound is put closer to the root. A low-diameter tree is hence formed.

And also, proposed several techniques to reduce the measurement cost for topology inference:

We note that there is high measurement redundancy among trace route results. A router or a link may be discovered multiple times in different trace routes. We integrate the Doubletree algorithm into our scheme to reduce the redundancy.

We compress trace route results to reduce bandwidth consumption for data exchange. Since routers are often repeatedly discovered in multiple trace routes, we set up a lookup table to map each router (that is, router IP and router name) into an integer. Later on, we represent routers by integers.

As mentioned, the computational complexity of Max-Delta is considerably high when the group size is large. We reduce it by conducting topology abstraction and reducing the computational frequency.

2.3 Previous Work on Network Topology Inference :

There are many ways of inferring a network topology. Network tomography techniques periodically send probing traffic and exploit the performance in correlation to infer network topologies. However, because the network properties measured (for example, loss rate or delay) are often unstable and inaccurate, it is difficult to infer an accurate topology. BGP routing tables can provide AS-level information, but they usually are not available to normal hosts in the Internet. We hence adopt trace route, which can obtain explicit router-level information by end hosts [10]. Trace route is implemented with Internet Control Message Protocol (ICMP) messages. Each time, the source sends out an IP datagram with a certain Time-to-Live (TTL) value to the

Destination. A router that receives the datagram is required to decrement the TTL by one before forwarding the packet. When a router receives an IP datagram who’s TTL is 1, it throws away the datagram and returns an ICMP “time exceeded” error message to the source. The returned Message contains the router’s name, IP address, and round-trip time (RTT) from the source. In another case, if the datagram arrives at the destination with an unused port number (usually larger than 30,000), the destination generates an ICMP “port unreachable” error message and returns it to the source. Therefore, in trace route, the source sends out a series of IP datagram’s with increasing TTL to the destination, and each datagram can identify one router in the path. The whole router-level path is hence identified. All the datagram’s of trace route are sent by UDP. An outgoing datagram in trace route contains 12 bytes of user data, 8 bytes of UDP header, and 20 bytes of IP header, for a total of 40 bytes. The size of the returned datagram’s changes. The returned ICMP message contains 20 bytes of IP header, 8 bytes of ICMP header, 20 bytes of IP header of the datagram that caused the error, and 8 bytes of UDP header, for a total of 56 bytes. For each TTL value, three ICMP messages are sent. According to these data, we can compute the network bandwidth consumed by a trace route.

Trace route-like tools have been widely used in Internet measurements such as Skitter, Mercator, and Rocket fuel .Skitter sends trace route packets from different locations worldwide to actively measure the Internet topology. Mercator utilizes a modified version of trace route to reduce the probing time. Rocket fuel combines information from BGP tables, trace routes, and Domain Name System (DNS) to infer Internet service provider (ISP) topologies. All these work focuses on Internet-level or ISP-level topology inference, and the major concern is how we can discover a complete network topology, including all the routers and links. However, in our study, we are only interested in the topology among a certain group of hosts that are arbitrarily distributed in the Internet. Furthermore, we only need an approximate topology, because most overlay applications are tolerant to a small distortion of the underlay topology. it is enough to discover around 93 percent of links for the FAT scheme to build a tree with low delay and low bandwidth consumption. Therefore, the key problem is how the measurement cost can be reduced while keeping high topology accuracy.

Barford et al. study the relationship between the measurement accuracy and the number of traceroute sources in topology inference. Given a list of destinations, they find that the marginal utility of adding additional traceroute sources declines rapidly after the

Second or third one. In other words, the first two or three sources can traceroute the destinations and discover the majority of the complete topology. Here, the complete topology is formed by the traceroute results from all the sources to all the destinations. In their experiments, the

Number of sources is much less than the number of destinations.

2.4 Review on Max-Delta:

Max-Delta considers how we can quickly and efficiently infer the router-level topology among a group of N hosts. As the router-level network is a sparse graph , it is anticipated that fully O(n2) trace routes among hosts are not necessary. Fig. 1 shows an example, where A, B, C, and D are hosts, and 1, 2, 3, 4, and 5 are routers. The dashed lines indicate the overlay paths between hosts. As the figure shows, paths A - B, A - D, and B - C have revealed all the underlay links and, therefore, we need to measure only three, instead of all the possible six, paths.

Fig. 1. An example of an underlay network. The dashed lines indicate overlay paths between hosts.

Max-Delta works as follows:

1) Hosts utilize lightweight tools such as GNP [28] or Vivaldi [29] to estimate their network coordinates and report them to a central server and

2) The following inference procedure is divided into multiple iterations. In each iteration, the server selects a target for each host to trace route. Hosts then trace route their targets and report the results to the server. The server then combines all the results obtained in the iteration, and based on that, it starts the next iteration on target assignment. Such a process is repeated until a predefined stop rule (for example, a certain number of iterations) is achieved.

In this paper, we combine our previous work to present a complete inference scheme with high scalability and efficiency.

2.5 Reviews on Doubletree:

Donnet et al. note that large-scale trace route measurements such as Skitter have high intra-monitor and inter-monitor redundancies [18], [19], [20]. That is, routers and links are

often repeatedly discovered in multiple trace routes. We take Fig. 1 as an example to show the following types of measurement redundancies:

1. Incriminator redundancy. Such a redundancy exists in trace routes from one monitor (that is, the trace route source) to multiple destinations. Suppose that A and B are two destinations and C is a monitor. When C conducts trace routes to A and B, C obtains two trace route paths: C – 4 - 3 - 1 - A and C - 4 - 3 - 2 - B. Clearly, routers 4 and 3 have been visited twice in the two trace routes. Incriminator redundancy hence occurs.

2. Inseminator redundancy. Such a redundancy exists in trace routes from multiple monitors to the same destination. Suppose that C and D are two monitors. When they conduct trace routes to the same destination A, they obtain paths C - 4 - 3 - 1 - A and D - 5 - 3 - 1 - A, respectively. In this case, routers 3 and 1 have been visited twice in the two trace routes.

Inseminator redundancy hence occurs. Donnet et al. have proposed a Doubletree algorithm to

reduce the redundancies. As discussed, a normal trace route sends a series of IP datagrams with increasing TTL to the destination. The TTL values start from 1. Doubletree modifies the working process of trace route. Given a source and a destination, a trace route in Doubletree starts probing

With TTL = h, where h is a predefined system parameter. The probing then proceeds toward the destination and backward to the source. Both the backward and forward probing use stop sets.  The one for backward probing, called the local stop set, consists of all the routers already

Discovered by the source. The forward probing uses the global stop set of (router; destination) pairs accumulated from all sources. A pair enters the global stop set if the router is visited in a trace route toward the corresponding destination. In either case, the probing stops whenever a

Member of the stop set is met.

2.6 Software and Hardware Specifications:

2.6.1 Software Specification:

Language                   :   Java

Front End Tool          :   Java Swing

Operating System      :  Windows 2000 or XP or LINUX

2.6.2Hardware Specification

Processor                  :    Intel[R] Pentium

RAM                        :    128MB

Hard Disk                 :   20GB